1—Safety and Microscopes

Pre-Lab

LEARNING GOALS

By the end of this unit, you should be able to do the following:

- List the six most important lab safety rules.

Focus a microscope on a slide at scanning power, low power, and high power.

Use the diaphragm to adjust the light levels for color or contrast.

Put the microscope away correctly.

For our first exercise, you will be reviewing basic lab safety rules and learning how to use a microscope. You will learn more about the microscope in the following lab, but for today we will just learn basic use.

LAB SAFETY

While each lab class may have rules specific to that lab, the following safety rules are universal in any lab containing chemicals. These rules must be followed at all times and may appear on tests and quizzes. Repeated violations of safety rules may be treated as disruptive behavior.

No food, drink, cosmetics, lip balm, or hand lotion may be used or stored in lab.

All items on this list must be stored inside your bag.

Nothing should go in your mouth or touch your face in lab.

Do not taste chemicals, stick pens in your mouth, or rest your hands on your face.

Wash your hands every time you leave the lab.

This is not just a rule for the end of the class period.

🩴 No sandals, flip-flops, or bare feet.

Your toes must be completely covered by your shoes. (Wearing socks with sandals is not sufficient.)

🫗 Tell your instructor about any spills or broken glass.

Your instructor will assist you in cleanup. Broken glass goes in a special container.

‍   Ask your instructor before pouring anything into the sink.

You may pour pure water down the drain, but everything else requires approval from your instructor.

SAFETY DOCUMENT

Your instructor will direct you to the full lab safety document in your iCollege section. You must read this document, sign the first page, and return it to your instructor by the next lab.

BASIC MICROSCOPE USE

We will go over every part of the microscope in detail next lab. For this lab, use the picture below as reference when your instructor mentions a part of the microscope.

Notes

Demonstration

BASIC MICROSCOPE USE

1.	Follow your instructor’s directions for how to properly remove a microscope from the storage cabinet.

2.	Plug in the microscope and turn on the light. Make sure the light dial is turned up all the way.

3.	Turn off the light, unplug the microscope and return it to the storage cabinet following your instructor’s directions.

4.	Now that you know how to correctly handle a microscope remove one from the storage cabinet correctly. Plug it in and turn it on.

Sample 1—Pre-Made Slides

1.	Get a pre-made Leaf Epidermis or Onion Root slide.

2.	Hold the slide up to the light and see the root pieces or leaf square.

3.	Place the slide on the stage in between the stage clips. Make sure the roots or dots are in the center of the light.

4.	Follow your instructor’s directions to close the diaphragm completely.

5.	Use the coarse adjustment knob to raise the stage all the way up. This is the most important part of using the microscope.

6.	Slowly rotate the coarse adjustment knob to bring the stage down while looking through the eyepieces.

7.	Keeping rotating the coarse adjustment knob until the image is visible and mostly in focus.

8.	If the image is not in perfect focus, slowly rotate the coarse adjustment knob back and forth to move the image in and out of focus.

9.	Rotate the objectives to the low power objective and look again through the eyepieces.

10.	Use the fine adjustment knob to move the image into focus.

11.	Find a student with a different slide and look through their eyepieces.

12.	Place your microscope back on the scanning power objective and return the slide to the instructor.

Sample 2—Wet Mount of Elodea

1. 	Take one small Elodea leaf and place it on a clean glass slide.

2. 	Add one drop of water and cover with a coverslip. (Make sure to remove the paper from the coverslip if present.)

3. 	Follow the same steps as in Sample 1 to focus your slide on the scanning power objective, then on the low power objective.

4. 	Once you have a good, focused image on the low power objective, your instructor will tell you how to switch the microscope to the high power objective.

5. 	Key rule: Never, ever, EVER use the coarse adjustment knob to lower the stage when switching to high power. You will lose focus and have to start all over. Don’t worry if you think there “isn’t enough room.” The high power lens compresses, so you can’t break it.

6.	If your image is out of focus on the high power objective, use the fine adjustment knob.

7. 	Place your microscope back on the scanning power objective, lower the stage, return the slide to the instructor, and return the microscope to the cabinet.

Let’s Review

Before finishing this lab, take a moment to reflect and review. Answer these review questions to recap key points:

- What items should you have out at your lab workspace for any given week? What things should be put away at all times during lab?

- When are goggles and gloves required in this lab? When are closed-toed shoes required?

- What things do you need to remember to do at the end of an experiment and/or class?

- Which objective lens is always used first on a microscope to get your specimen in focus? Which adjustment knob is always used with that objective lens?

- What does the diaphragm of the microscope do?

- Which objective lens should the microscope be on when you remove a slide from the stage?